# MOTOR MOUNTAIN MASTERS MODIFIED RULES

Jennerstown Speedway 
(Intersections of Rt. 30 & Rt. 985)
PO Box 270
206 Somerset Street
Jennerstown, PA 15547-0099

Track Phone: 814.703.8009
Fax: 814.703.8163

For scheduling, tickets and race related questions:
Email: billhribar@jennerstownspeedwaycomplex.com

For rule clarification questions:

Email: Wcbauer@zoominternet.net

Table of Contents

MOTOR MOUNTAIN MASTERS MODIFIED RULES	1

PREFACE	3

1.	TRACK COMMUNICATION:	4

2.	ENGINE:	4

2.1	 ENGINE SET BACK:	4

2.2.	ENGINE OPTIONS:	4

3.	WEIGHT/ENGINE RPM REQUIREMENTS:	5

4.	EXHAUST SYSTEMS:	5

5.	IGNITION SYSTEM:	6

6.	COOLING SYSTEM:	6

7.	BATTERY/STARTER:	6

8.	AIR FILTER:	6

9.	FUEL CELL/FUEL PUMP:	6

10.	FUEL:	6

11.	CHASSIS:	6

12.	SUSPENSION/WEAREND:	7

13.	SPINDLES:	7

14.	BODIES:	7

15.	BODIES: (FIGURE #1):	7

BODIES FIGURE #1	9

16.	FRONT BUMPER:	9

17.	REAR BUMPER:	9

18.	NERF BARS:	10

19.	WINDOWS:	10

20.	EXOTIC MATERIALS:	10

21.	STEERING:	10

22.	BRAKES:	10

23.	SHOCKS & SPRINGS:	10

23.6.	RUSH Bilstein Bandit Shock Package "Option" 75 lbs. weight reduction	10

24.	WHEELS:	11

25.	TIRES:	11

26.	RADIOS & MIRRORS:	11

27.	TRACTION CONTROL DEVICES:	11

28.	DRIVE LINE:	11

29.	ROLL CAGE:	12

30.	PAINT & NUMBERS:	12

31.	SAFETY:	13

32.	RULE BOOK DISCLAIMER:	14

# PREFACE

The rules and/or regulations set forth herein are designed to provide for the orderly conduct of racing events and to establish minimum acceptable requirements for such events. These rules shall govern the condition of all events, and by participating in these events, all participants are deemed to have complied with these rules. No expressed or implied warranty of safety shall result from publications of or compliance with these rules and/or regulations. They are intended as a guide for the conduct of the sport and are in no way a guarantee against injury or death to a participant, spectator or official. The race director shall be empowered to permit reasonable and appropriate deviation from any of the specifications herein or impose any further restrictions that in his opinion do not alter the minimum acceptable requirements. No expressed or implied warranty of safety shall result from such alteration of specifications. Any interpretation or deviation of these rules is left to the discretion of the officials and their decisions are final.

Jennerstown Speedway Complex 2018 General Rules will apply for this event unless specifically specified my Jennerstown Speedway management.

# TRACK COMMUNICATION:

Optional Race Control RACEceivers frequency 454.000.

# ENGINE:

## 2.1	 ENGINE SET BACK:

Motor must be a minimum of 72-inches from the center of the rear end to the rear of the engine block

## 2.2.	ENGINE OPTIONS:

## GM Engine Part Number #88958602/19258602 Option #1:

      - All engines are to remain sealed from the factory. These engines are sealed at the intake manifold, cylinder head, front cover and oil pan with a special GM twist off bolt. The original factory seals (bolt-type) must remain unaltered.

      - RUSH & Crate USA seal tags permitted.

      - Only GM crate engine specific valve springs may be used for replacement and/or repair, Part Number # 10212811 for singles or part number 19154761 for kit of 16 springs. Valve springs may be shimmed to GM manual specs only.

      - The only change from production will be that of replacement oil pan Moroso p/n 21319 and matching oil pump p/u assy. Before mentioned oil pan is the only legal replacement permitted.

## GM Crate Engine Carburetors:

      - Holley 650 HP #4150-80541, Holley 650 HP #4150-80541-1, or Holley 650 HP #4150-80541-2 carburetors permitted.

      - Must be gauge legal.

      - Carburetors must have 1 11/16" base plate maximum

      - Carburetor venturi size 1.248-1.252.

      - All carburetors in competition must retain conventional style floats along with needles and seats.

      - Maximum allowed fuel pressure is 12 psi.

      - May use one carburetor spacer (1" - one piece with 0.040" tolerance maximum) and two standard paper gaskets (maximum 0.065" thick), one gasket between intake to spacer and one gasket between spacer to carburetor. Carburetor spacer may not protrude down into intake manifold.

      - Engine must be naturally aspirated.

## Ford M-60070-347JR- Option #2:

      - All engines are to remain sealed from the factory as produced by Ford.

## Ford Engine Carburetors:

      - Holley 650 HP #4150-80541, Holley 650 HP #4150-80541-1, or Holley 650 HP #4150-80541-2 carburetors permitted.

      - Must be gauge legal.

      - Carburetors must have 1 11/16" base plate maximum

      - Carburetor venturi size 1.248-1.252.

      - All carburetors in competition must retain conventional style floats along with needles and seats.

      - Maximum allowed fuel pressure is 12 psi.

      - May use one carburetor spacer (1" - one piece with 0.040" tolerance maximum) and two standard paper gaskets (maximum 0.065" thick), one gasket between intake to spacer and one gasket between spacer to carburetor. Carburetor spacer may not protrude down into intake manifold.

      - Engine must be naturally aspirated.

      - Track issued 1.45” restrictor plate must be installed.

## Custom Built/Open Engines – Option #3

      - Must be small block engines.

      - Maximum Chrysler engine displacement of 366 cid.

      - Maximum GM engine displacement of 355 cid.

      - Maximum Ford engine displacement of 356 cid.

      - No Ford 9.2 blocks allowed

      - Must be stock cast iron blocks with all standard internal and external dimensions except the allowable maximum overbore.

      - Steel crank and rods only.

      - Compression ratio may not exceed 10.8/1 as computed by the track KATECH whistler. NO TOLERANCE. Official’s decision is FINAL.

      - No dry sump oiling permitted.

      - Cylinder Heads:

      - Cast iron stock heads only. No modifications.

      - Only steel valves and valve springs permitted. Stud mounted roller rocker arms permitted, maximum 1.65 to 1 ratio.

## Open Motor Carburetor:

      - Holley 2300 2bbl. 500 CFM carburetor, model #4412, including HP

      - Must be gauge legal

      - Absolutely no modifications permitted except tuning of jets, power valve, or bleeds.

      - All parts must be original Holley replacement parts.

      - No polishing, grinding or drilling of holes permitted.

      - ONLY a Moroso #64966 un-modified carburetor adapter permitted.

# WEIGHT/ENGINE RPM REQUIREMENTS:

  - GM Engine Option #1: Min weight is 2500 lbs, 6300 RPM, Max left side weight is 59%

  - Ford M-60070-347JR Option #2: Min weight is 2500 lbs, 6000 RPM, Max left side weight is 59%

  - Custom Built/Open Engines Option #3: Min weight is 2550 lbs, 6000 RPM, Max left side weight is 59%

  - All weights are AFTER the race with driver.

  - RUSH Bilstein Weight Break:  A 75 lbs weight break will be allowed for the RUSH Bilstein Bandit Shock Package (See Rule 23.6 Shocks)

# EXHAUST SYSTEMS:

  - Exhaust systems must be mounted in such a way to direct spent gases away from areas of possible spillage of fuel.

  - Headers may be used but must be pointed away from ground.

# IGNITION SYSTEM:

  - Any battery powered ignition system produced for highway use is allowed.

  - No crank trigger systems, no magnetos, boosters or other devices allowed.

# COOLING SYSTEM:

  - No anti-freeze permitted.

  - Only belt driven block mounted OEM similar design water pumps permitted.

  - Belt driven fan blade or electric cooling fan are permitted.

# BATTERY/STARTER:

  - Maximum one 16-volt battery.

  - Batteries must be securely fastened outside driver's compartment.

  - All cars must have a starter in working order.

  - A main electrical cut off switch mounted in the driver's compartment must be clearly marked and easily accessible to driver and safety crews.

# AIR FILTER:

  - All air must enter engine through top of carburetor ONLY.

  - Air filter maximum size 16-inches outside diameter by 4-inches high.

# FUEL CELL/FUEL PUMP:

  - All cars must have a 32-gallon maximum fuel cell consisting of a rubberized bladder with a fill plate that meets or exceeds FT3 specifications housed in a minimum 22-gauge steel container.

  - The entire container must be visible for ease of inspection.

  - The fuel pick up must have a check valve.

  - Mechanical fuel pumps only; must be in stock location.

# FUEL:

  - Only Sunoco Racing Fuel - Standard (commonly referred to as 110 and/or purple) or RUSH ‘91’ will be the specified fuels permitted for competition in RUSH Racing Series sanctioned events.

  - Additives and/or blending of any type including; methanol, alcohol, nitrous oxide, propylene oxide, nitromethane or other performance enhancing chemical additives will not be permitted.

# CHASSIS:

  - Factory production full 1950 or newer perimeter American passenger car frames and Howe racing tubular 358 AMOD Impala Modified Front frames permitted.

  - No Jeep, Bronco, or 4-wheel drive, Sports car, etc. frames permitted.

  - No front-wheel drives permitted.

  - Frame may be cut in rear only at point not further than 36-inches from center of rear end housing.

  - Maximum tread width front and rear is 76 inches, measured from bead flange to bead flange on the outside of wheels, the front is measured at the front of the wheels, and the rear is measured at the rear.

  - Track width will be measured at hub height.

  - The track gauge is the final word.

# SUSPENSION/WEAREND:

  - Front suspension must be in stock location.

  - Lower A-frames may be OEM, OEM replacement or approved steel or aluminum tubular construction type.

  - Tube type steel or aluminum upper A-frames with or without adjustable heim joints are allowed.

  - Heim ends are permitted.

  - Any 3 or 4 link rear suspension system permitted.

  - All links must mount at one point on rear axle and one point on frame.

  - Rear ends may be Ford style 9-inch floater or quick change type.

  - No driver adjustable suspension or links permitted.

  - No hydraulic, ratchet, or electric weight jacks permitted anywhere in or on car.

  - Minimum wheelbase of 108-inches.

## SPINDLES:

  - Stock passenger car spindles and OEM equivalent safety spindles permitted.

  - No fabricated spindles.

  - No alterations (welding or heating) allowed on the spindle arms.

# BODIES:

  - Any Bodies that are not in compliance with the following rules may request official approval.

  - Bodies may be pre-bought aftermarket panels of recognized economy Modified styles or homemade reproductions thereof in accordance to IMCA body rules listed below.

  - Must be full size roof with full A pillar.

  - Rear side windows must be either open or clear lexan.

  - Windshield optional.

  - Firewalls and floorboards are mandatory.

  - Fiberglass or metal duplicates of body parts are permitted.

  - Handmade body parts may be constructed of steel, aluminum or fiberglass

  - Rear Deck Spoilers and rear lexan windows Are Permitted. Maximum 5 “ spoiler

  - No ground effects allowed inside or outside the car.

  - Roof height must be a minimum of 42-inches.

  - Engine compartment will remain open.

  - No engine side panels.

  - Maximum rear spoiler height of 5 inches

# BODIES: (FIGURE #1):

- 12″ min 24″ max

- The maximum roof height of 54” inches. 5″ max slope.

- The sides of the engine compartment will remain open. Any side panels in the engine compartment area will not be permitted. A six-inch drop on either side of the hood will be permitted and it must be enclosed at the rear of the hood. Hood must slope towards the nose of the car (No Reverse Rake to the Hood).

- The top of the doors must not extend forward of the rear engine plate. The bottom of the doors may extend a maximum of 12” inches past the motor plate toward the front of the vehicle.

- Front roof post width 16″max at the bottom. Roof post at the bottom rear must then go upward at a 90 degree angle. The front roof post must be in a straight line from top to bottom and must come down to the deck with a max 2″ step (P).

- Wheelbase: Minimum 108″, maximum 112″.

- Door panel 32″max 26″min.

- 72″ max.

- 12″-18″ door opening height.

- With a level, must have no more than 2″ clearance at rear roof, and 5″ at top front. Original roof line of vehicle must be maintained with a maximum of five (5) inches of slope measured with a level from highest point minus the stiffener to lowest point. No more than one-half (1/2) inch stiffener allowed at the rear of the roof.

- Maximum deck length 120”.

- Rear corner panel length from center of hub to back 48″ max, 34″ min.

- 7″ max at top rear, gradual slope from roof to this point when using sail panel as spoiler side with max spoiler height of 5”. The spoiler may be fabricated from lexan or aluminum. The Sail panels must be the same design and size on each side (Both open with same opening dimension or both closed). Maximum bow of sail panel is 4”. 8″ max spoiler allowed if the sail panel remains separate from the spoiler sides. When using 8″ spoiler, sail panels can be no higher than 4″ from deck at end point, spoiler supports shall be same height as spoiler at rear and 4” tall max at front and 24” min total length, sail panel may not be farther back than leading edge of spoiler support on 8” spoilers. Maximum spoiler width will be 68” on 5” or 8” spoilers.

- Interior slope is 4″ max from front to rear and flat across and must maintain same slope from front to rear. You must have used 50% of your 4″ max slope at the half way point on the deck.

- Maximum height of 38” inches. The top of the interior must be flush with the top of door. No Drop Decks

- Maximum width of nose at any point will be 45”. The nose piece shall not be attached any further back than the radiator. The MD3 modified nose and Dominator nose will be legal. All aluminum fabricated nose pieces can have a 2 1/2” lip on each side of nose piece (length of the nose). The nose piece must have a minimum of four (4”) inches from the ground to the bottom edge of the nose piece. Noses must be flat from top to bottom (No curving of nosepiece).

- Hood max width is 68″ max 24″min. No narrower than radiator.

- The maximum roof width will be 52” inches with a minimum of 44” inches.

- The maximum roof length will be 57” inches with a minimum roof length of 40” inches

- On the left side the rear bottom of the door may flare out six (6”) inches for tire clearance. The bottom of the flare must be curved and rounded not pointed in any fashion. No wheel skirts of any type will be allowed.

- The maximum body width at the body’s widest part will be 72” inches. The maximum deck width will be 68”. Rear wheel openings must remain same on both sides of car and maintain a minimum 2” wheel clearance

# BODIES FIGURE #1

# FRONT BUMPER:

  - Bumper must be leading edge of the car

  - Must be mounted to end of frame, with bottom loop parallel to the ground.

  - Minimum 1-1/4″ tubing must be used.

  - Bumper cannot extend more than 42” from center of front wheel.

  - Farthest point of nose/bumper cannot be more than 42” from center of front wheel hub.

# REAR BUMPER:

  - No sharp edges.

  - Rear bumper may be tubing or square stock and must protect fuel cell.

  - Center of bumpers must be at least 18″ from ground.

# NERF BARS:

Must be inside of line from rear tire to front tire with no sharp edges.

# WINDOWS:

19.1	Driver's side window and passenger side window must remain open except for support angles in roof pillar openings.

19.2 	Support angles cannot extend past the front roof line. Roof pillars must be in place.

- EXOTIC MATERIALS:

  - All exotic materials are illegal, which includes titanium, tungsten, and/or carbon fiber.

  - As noted above, carbon fiber driveshaft’s are legal for safety purposes.

# STEERING:

  - Steering box must be OEM.

  - No center steering.

  - No rack and pinion.

  - Steering may be modified to suit driver’s taste, but must be on left side of cockpit.

  - Steering wheel must be equipped with a center “nose pad”.

# BRAKES:

Must have brakes on all four wheels and lock up on inspections.

# SHOCKS & SPRINGS:

  - One steel body shock per wheel; shock body must be one piece all steel.

  - No Schrader valves.

  - Shocks may have no external means of adjustment; non adjustable.

  - No bump stops, Belleville washers, or bump springs permitted.

  - One spring per wheel, no stacked springs. Coilovers permitted.

### RUSH Bilstein Bandit Shock Package "Option" 75 lbs. weight reduction

    - Cars must be equipped with RUSH Bilstein Bandits on all four corners; one shock, per wheel permitted to receive weight break. Cars found with non-RUSH Bilstein Bandits Shocks must weigh under standard shock rule weight or will be disqualified from the event.

    - Only RUSH Bilstein Bandit Series shocks by Bilstein or CR USA shocks by Bilstein may be used in competition to be permitted to compete at the designated Bilstein Bandit weight break (GM or Ford).

    - In order to further insure that all competitors are racing on unaltered shocks, competitors agree to allow race car to be inspected by Track Officials at any time and for any reason, and agree to immediately relinquish any one or all shock(s) from any position or all positions on his or her race car to tech officials as a swap. Competitor must accept from tech officials a shock(s) of the same valving part number in return for the shock(s) relinquished. Refusal of a driver to accept the “swap” will disqualify the driver from the night's events.

    - RUSH Bilstein Bandit Shock Package "Option" consist of eight (8) shocks with the following part numbers:

      - RUSH RF 12-2 or CRUSA RF 12-2, RUSH RF 9-3 or CRUSA RF 9-3, RUSH RF 7-3, RUSH 16-2, RUSH LF 2.5-2.5 or CRUSA LF 2.5-2.5, RUSH 4-3, RUSH 5-2, RUSH 6-1.5 (Note: Shocks are sold individually).

# WHEELS:

Must be steel; 15” x 8”.

# TIRES:

  - The Official track tire will be the American Racer:

  - Right Sides 27.0/8.0-15T AR 970

  - Left Sides 26.5/8.0-15T AR 970

  - No tire bleeders.

# RADIOS & MIRRORS:

  - Radios permitted.

  - Mirrors permitted.

  - Optional Raceceiver Race Control frequency 454.000

# TRACTION CONTROL DEVICES:

All Traction Control Devices are strictly prohibited

# DRIVE LINE:

  - OEM production type 3 or 4 speed manual transmissions and automatic transmissions with either torque converter or direct drive couplers are acceptable.

  - Bert/Brinn style internal wet disc clutch transmissions are acceptable and must bolt to the back of the engine as designed.

  - A minimum 5.5 inch diameter metallic multi-disc external racing clutches is permitted.

  - No carbon fiber or non-standard type material will be permitted.

  - The clutch must bolt to the crankshaft as designed and must be housed in a steel, aluminum, or magnesium 360 degree bell-housing bolted to the back of the engine.

  - A 1-inch inspection hole must be drilled in the bottom of the bell-housing directly under the clutch unit.

  - No slipper-style or centrifugal type clutches will be permitted.

  - All friction plate cores and floating plates must be made of metallic steel.

  - No 5-speed transmissions permitted.

  - No in and out boxes.

  - No buttons.

  - Manual transmissions must be clutch operated.

  - Clutch, torque converter or direct coupler must be inside of bell-housing or transmission housing.

  - Stock OEM transmissions may use either stock OEM production clutches or triple disc with minimum 7-inch disc diameter.

  - Steel discs, pressure plates, flywheels and flex plates only, no aluminum flywheels or flex plates.

  - Stock size and style single disc clutch assembly must be equipped with an explosion proof steel bell-housing.

  - All transmissions must have at least two forward gears and one reverse gear and must have neutral gear.

  - All race cars must be able to join the lineup on demand unaided or go to rear of race.

  - No carbon fiber drive shafts. Steel or Aluminum drive shafts are acceptable.

  - Driveshaft must be painted white.

  - Minimum of 2 driveshaft loops are required and must be constructed of at least 3/16” x 2” steel.

  - One must be mounted no more than 6-inches back from front of driveshaft and the second half way back driveshaft.

# ROLL CAGE:

  - Roll cage must consist of continuous hoops not less than 1-3/4 inch outside diameter, and must have a wall thickness of a least .095.

  - Must be frame mounted in at least 6 places.

  - Cages may be sonic tested if deemed necessary.

  - Must consist of a configuration of front and rear hoops connected by tubing on the side or side hoops.

  - Driver’s head must not protrude above roll cage with helmet on.

  - Roll cage must be securely supported and braced.

  - Top hoop must have crossbar.

  - Protection of feet is mandatory.

  - Bar across back of engine with vertical bars and rub rails, or similar protection.

  - No brace bars forward of a cage may be higher than stock hook height.

  - Driver door bars must be as parallel with the ground as possible and located perpendicular to the driver so as to provide maximum protection for driver, but without causing difficulty in getting in or out of vehicle.

  - The side bars must be welded to the front and rear of the roll cage members.

  - Door bars must be at least 1-3/4 Outside Diameter .095 thickness, and have 4 bars.

  - Driver side door cage must be plated with 1/8 inch steel from top door bar down to frame, must be welded or bolted on outside of cage.

  - Front bumper must be mounted frame end to frame end, with the bottom loop parallel to the ground. Minimum 1-1/4 inch pipe able to support car if lifted by wrecker. Rear bumpers may be constructed of pipe or flat stock, and must protect fuel cell. Center of bumpers (front and rear) must be 18-inches from the ground. (2-inch tolerance). Rear bumpers, nerf bars, and bodies must not extend beyond width of rear tires and must not contain any sharp edges.

# PAINT & NUMBERS:

  - Car numbers must appear on both doors and roof of car.

  - Each number must be a minimum of 24” high.

  - Roof number must read from scorers stand.

  - Minimum 4” number must appear on upper right hand corner of windshield and in right rear tail light area. No Duct Tape.

  - All numbering jobs must meet with scorer’s approval.

  - Single or double digit numbers only.  No letters or triple digits permitted.

  - Contrasting solid color numbers only.

  - All car numbers must be registered with Jennerstown Speedway.

  - All returning drivers in the same division will have thirty (30) days from the date of the registration event to register their number.

  - All cars must be neat in appearance.

# SAFETY:

### General

    - IT IS THE RESPONSIBILITY OF THE DRIVER, Not the TRACK OFFICIALS, OR THE PROMOTER TO INSURE HIS/HERS PROTECTIVE CLOTHING, SAFETY RESTRAINT SYSTEMS, SEAT AND ALL OTHER SAFETY ITEMS INSTALLED CORRECTLY, PROPERLY MAINTAINED AND USED.

    - Must have all standard safety equipment including two (2) drive shaft safety hoops.

### Helmets

    - All driver’s will wear a full-face helmet that meet a minimum safety rating of Snell SA2010/FIA-8860, Snell SA2015/FIA-8860 or SFI 31/1/2015

    - Head and Neck restraints required. The head and neck restraint system should be mounted and connected to the helmet per the helmet manufacture and head and neck manufacture instructions.

    - Arm restraints are recommended and should be connected and used as instructed by the manufacture.

### Seat Belt/Restraint System

    - Minimum 5-point seat & shoulder harness system minimum 3” wide and maximum required

    - Each car should be equipped with minimum of an SFI 16.5 or SFI 16.1 approved restraint system, until the date of the belt expiration (two years from the date of manufacturer). Seat belt restraint systems shall be installed and used in accordance with manufacturer’s instructions.

    - Seat belt material should not be permitted to come in contact with any sharp or metal edge, including when the material passes through the seat metal.

    - Rolled and/or de-burred and/or flanged edges or anywhere seatbelt webbing passes though and may come in contact with abrasive edge are recommended.

### Cockpit Window Net

An approved driver’s cockpit net required. The net must be a minimum of three ribbons and meet SFI 37.1 specifications, installed per the manufactures specifications.

### Protective Clothing

    - All drivers will be required to wear a fire resistant driving uniform meeting minimum of the SFI 3.2A/5 specification and display a valid SFI 3.2A/5 label.

    - Drivers must wear gloves at all times they are on track during practice and competitions.

    - Driver’s gloves must meet or exceed the SFI 3.3 specification and have a legible and valid SFI 3.3 label.

### Seat

    - All cars must be equipped with an aluminum safe racing seat, high back type recommended. Aluminum seats with an FIA and/or SFI 39.2 rating are recommended.

    - All seats must be mounted to the frame as required by the seat and chassis manufacturer. Full containment seat(s) and/or aftermarket bolt on head restraints are recommended.

    - All areas surrounding the head should have padding.

    - The driver’s seat must be securely fastened to the frame and/or cage in six positions with a minimum if six (6) 3/8”-inch bolts. There should be a minimum of four (4) bolts on the bottom and two (2) in the back of the seat. All seats must be a minimum of 1/8”-inch magnetic steel plate under and up the back 4” inches and be as wide as the seat.

    - The seat must be positioned completely to the left of the centerline of the car (no center steering) on the driver’s side.

    - NO FIBERGLASS SEATS.

## Safety Other

    - Fire extinguisher must be a minimum of 5 lbs. and be mounted securely within driver’s reach and be fully charged.

    - An approved head and neck restraint system is HIGHLY recommended.

    - A neutral safety switch or brake depression switch is mandatory.

    - Only SFI frame retardant seat, roll bar, knee and steering pads and/or padding should be utilized.

# RULE BOOK DISCLAIMER:

  - The rules and/or regulations set forth herein are designed to provide for the orderly conduct of racing events and to establish minimum acceptable requirements for such events. These rules shall govern the condition of all events, and, by participating in these events, all participants are deemed to have complied with these rules.

  - No expressed or implied warranty of safety shall result from the publication of or compliance with these rules and/or regulations. They are intended as a guide for the conduct of the sport and are in no way a guarantee against injury or death to a participant, spectator, or official.

  - The race director shall be empowered to permit reasonable and appropriate deviation from any of the specifications herein or impose any further restrictions that in their opinion do not alter the minimum acceptable requirements.

  - No expressed or implied warranty of safety shall result from such alterations of specifications. Any interpretation or deviation of these rules is left to the discretion of the officials. Their decision is final. On occasion when situations arise that are not covered by written rules, the track officials may put special rulings into effect. Once such rulings are acted upon, they may become an act of policy and will be added to the existing rules of procedures.

The Management of JENNERSTOWN SPEEDWAY COMPLEX, LLC